1. Accelerometer (top plot)

-The lines ax, ay, az represent motion along 3 axes.

-The red line |a| = overall acceleration magnitude.

-At around 6–7 seconds, there’s a huge spike followed by sudden stability → this is the impact of the fall followed by lying still.

2. Gyroscope (bottom plot)

-Lines gx, gy, gz are angular velocities around 3 axes.

-Red line |g| = overall angular velocity magnitude.

-Same spike around 6–7 seconds = rotational movement of the body during the fall.

✅ Why this is important

-This confirms the dataset is correctly loaded and meaningful.

-You can clearly see the signature of a fall:

-Sudden burst in both accelerometer & gyroscope

-Followed by near-zero (lying still)

-This is exactly the type of pattern our deep learning model will learn to detect.

In our project, you have accelerometer and gyroscope data from two types of files:

1. Fall activity files -4 data collected when a fall actually occurred.

2. Daily life activity files -+ data collected during normal movements like walking, sitting,

standing,

bending, etc.

What we Actually Did in This Process:

By comparing fall and daily-life files using accelerometer and gyroscope readings, you have

essentially

performed data analysis and pattern differentiation — the foundational step of fall detection.

Let's see what this means practically.

Step-by-Step Meaning

1. Data Observation and Feature Understanding

You observed how the sensor values (X, Y, Z) of.

• Accelerometer (which measures acceleration or sudden movement),

• Gyroscope (which measures angular rotation or orientation),

differ between normal activities and falls.

Achievement: We’ve learned the unique patterns or threshold ranges of sensor values that signify a fall.

2. Feature Extraction / Comparison Output

While comparing both files, you might have computed:

• Mean, Variance, Standard Deviation of accelerometer and gyroscope values.

• Resultant Acceleration (using M (ax2 + ay2 + az2)).

• Angular velocity changes from the gyroscope.

By comparing these, you likely observed that:

• In normal activities, acceleration changes smoothly.

• In falls, acceleration and angular velocity show sudden spikes.

Achievement: We verified that the sensor data can effectively distinguish a fall event from other daily activities — proving

our data is valid and usable for the detection model.

3. Pattern Differentiation (Core Insight)

You successfully differentiated between:

• Normal patterns (continuous, stable, low acceleration and angular change)

• Fall patterns (sudden high acceleration + high rotation followed by stillness)

Achievement: You verified that the sensor data can effectively distinguish a fall event from other daily

activities — proving your data is valid and usable for the detection model.

4. Output Interpretation

When you compared both datasets, your output might have shown:

• Graphs where fall data have sharp peaks or spikes.

• Tables where average accelerations or angular velocities during falls are higher.

• Thresholds or classification indicators separating the two.

Achievement (based on output):You established a baseline or threshold-based understanding of how falls differ —

which will be later used for.

• Rule-based detection (if value > threshold fall)

• Or machine learning classification (training a model with these features)

Summary:

By comparing the fall and daily life activity files through accelerometer and gyroscope, you identified and verified distinct sensor patterns that separate falls from normal movements.

This process allowed you to extract important features, understand the behavior of both sensors, and establish a reliable foundation for building your fall detection algorithm.