System architecture — IoT-based Fall Detection using LSTM

1. Executive summary (short paragraph to open the section)

The proposed system is an IoT-enabled fall detection pipeline that combines wearable inertial sensing with a trained LSTM classifier to detect fall events in near real-time. Raw accelerometer and gyroscope data are windowed and preprocessed on-device or at the edge, passed to a lightweight LSTM model for inference, and when a fall is detected the system triggers local alerts and remote notifications. The architecture is modular—separating sensing, preprocessing, inference, communication, and alerting—so it can be deployed fully on an edge device (Raspberry Pi / Jetson Nano), partially on a microcontroller with cloud inference (ESP32 + cloud), or hybrid (on-device inference with cloud logging).

2. High-level components (bullet list)

- Wearable Sensor Node

  - IMU (accelerometer + gyroscope) such as MPU6050/MPU9250.

  - Microcontroller: ESP32 (preferred) or Arduino Nano 33 BLE Sense.

  - Responsibilities: sampling, buffering, optional local pre-filtering, and sending windows or raw stream.

- Edge Processor / Gateway (optional)

  - Raspberry Pi / Jetson Nano / small laptop.

  - Responsibilities: final preprocessing, run LSTM inference (PyTorch/TFLite), logging, local alerting (buzzer/LED), and forwarding events to cloud.

- Inference Module (Model)

  - Trained LSTM model (input: 512 timesteps × 6 features; output: probability of Fall / ADL).

  - Format: PyTorch (during research), converted to ONNX / TensorFlow Lite for edge deployment.

- Communication Layer

  - Protocols: MQTT (lightweight), HTTP REST, or WebSocket.

  - Topics / endpoints for sensor telemetry, prediction events, and device management.

- Cloud Backend (optional)

  - Storage for telemetry, model versions, dashboards, and alert distribution (SMS/email).

  - Services: MQTT broker (Mosquitto), database (InfluxDB / PostgreSQL), and notification service (Twilio / Firebase).

- User Notification & Dashboard

  - Mobile app / web UI to display alerts, timelines, and model confidence.

  - Alert channels: SMS, email, push notifications, or local buzzer.

- Management & Update Service

  - Model deployment manager (push updated .tflite or .onnx), OTA firmware updates.

3. Detailed dataflow (text + numbered steps)

- Sensing

  - IMU sampling at fs = 200 Hz (as in SisFall). Each sample is a 6D vector: [ax, ay, az, gx, gy, gz].

- Local Buffering & Optional Preprocessing (on MCU)

  - Buffer raw samples and optionally compute magnitude features (|a|, |g|).

  - Optionally perform light filtering (e.g., simple low-pass) to reduce sensor noise.

- Windowing

  - Sliding windows of WINDOW_SIZE = 512 samples (≈2.56 s) with STEP = 256 (50% overlap).

  - Option A: MCU sends completed windows to edge/cloud.

  - Option B: MCU sends raw stream and edge performs windowing.

- Normalization

  - Per-window zero-mean / unit-std normalization (same transform used at training).

- Inference

  - The LSTM model consumes (512, 6) windows and outputs logits or a sigmoid probability for Fall.

  - Threshold (e.g., p_fall >= 0.5) decides Fall vs ADL; threshold tuned on validation set.

- Decision & Alerting

  - If Fall detected, raise local alert (vibration/buzzer) and publish event to MQTT topic device/<id>/event with payload {timestamp, p_fall, file_id, meta}.

  - Optionally require k consecutive positive windows or majority vote across m windows to reduce false positives.

- Logging & Cloud Processing

  - Event and telemetry are stored for offline analysis; cloud may re-run inference for audit.

- Notification

  - Cloud or gateway triggers SMS/Push notifications with location and event details.

4. Component diagrams: textual specification to draw figure

Figure X — System Architecture (block diagram)

- Left: Wearable Sensor Node box (MPU6050 + ESP32) labeled: "Sampling @200 Hz → Buffer → (Optional) Windowing".

- Arrow → Gateway/Edge box (Raspberry Pi): "Windowing / Normalization → LSTM Inference (.tflite) → Local Alert / Publish".

- Arrow → MQTT Broker (Cloud) box: publishes device/<id>/telemetry and device/<id>/events.

- Arrow → Cloud Backend: DB + Dashboard + Notification Service.

- Arrow → User Devices: Mobile App / SMS / Web Dashboard.

Figure caption (for paper):
Figure X. System architecture for the IoT-based fall detection pipeline. Wearable IMU nodes stream motion data to a gateway that performs preprocessing and runs an LSTM-based classifier; detected fall events trigger local alerts and cloud notifications. The design supports both on-device and cloud-based inference.

5. Module-level design & interfaces

A. Wearable Node (ESP32) — Responsibilities & pseudocode

- Sensors: read ax, ay, az, gx, gy, gz

- Sampling loop: read sensor → push to circular buffer

- When buffer has WINDOW_SIZE samples: optionally compute per-window normalization and publish JSON or binary to gateway via MQTT or HTTP.

Pseudocode (microcontroller)

buffer = CircularBuffer(WINDOW_SIZE);

while(true){

sample = read_IMU(); // 6 floats

buffer.push(sample);

if(buffer.size() >= WINDOW_SIZE){

window = buffer.get_last(WINDOW_SIZE);

// optional lightweight normalization or feature computation

publish_mqtt("device/<id>/window", serialize(window)); // binary or base64

buffer.advance(STEP); // overlap handling

}

delay(5); // for 200Hz

}

B. Gateway (Raspberry Pi) — Responsibilities

- Subscribe to device/<id>/window

- Deserialize window → apply x = (x - mean)/std (same as training)

- Run .tflite model inference (TFLite interpreters)

- Apply decision rule and smoothing (e.g., require >=2 consecutive positive windows within 3s)

- Publish device/<id>/event with payload:

Json

{

"device_id":"dev001",

"timestamp":"2025-10-28T12:00:01Z",

"p_fall":0.87,

"decision":"FALL",

"window_start":123456,

"meta":{"subject":"SA01","file":"F01_SA01_R01.txt"}

}

If decision == FALL, trigger local alert and call cloud API for SMS.

C. Cloud API (optional)

- REST endpoints:

  - POST /api/v1/events — receive event (authentication token)

  - GET /api/v1/devices/<id>/events — query events

- MQTT topics:

  - device/<id>/telemetry — raw sampled metrics (light streaming)

  - device/<id>/events — event notifications

6. Model conversion & runtime formats

- Training: PyTorch .pth (research experiments)

- Export:

  - PyTorch → ONNX (for edge conversion)

  - ONNX → TensorFlow → TFLite (for microcontroller / Raspberry Pi using TFLite interpreter)

- Edge inference options:

  - Raspberry Pi / Jetson Nano: use PyTorch or ONNX runtime

  - Raspberry Pi OS (no GPU): TFLite or ONNX Runtime CPU

  - ESP32 / MCU: convert to TensorFlow Lite Micro (model must be quantized and very small — may require pruning or using a smaller model)

Note: LSTM is heavier to run on microcontrollers; alternatives:

- Replace LSTM with small 1D-CNN + depthwise separable convs for ultra-low-power device

- Use model quantization (8-bit) and pruning to reduce memory

7. Reliability & production considerations

A. Decision smoothing to reduce false positives

- Apply majority voting: a fall is confirmed only if >=k of last m windows predicted fall.

- Temporal rules: require a high probability event followed by reduced motion (|a| low) to confirm fall.

B. Latency

- Sampling (2.56 s window) + inference (~10-200 ms depending on hardware) → detection latency ≈ 2.6 s (configurable by window size).

- If lower latency required, reduce window size (tradeoff with accuracy).

C. Energy / Power

- MCU sampling at 200 Hz consumes power; duty-cycle to conserve battery (e.g., run inference less frequently when stable).

D. Privacy & Security

- Encrypt device ↔ gateway communication (TLS for MQTT over websockets or MQTT over TLS).

- Authenticate devices with per-device tokens.

- Transmit minimal data (e.g., events + short windows) rather than continuous raw streams to preserve privacy.

8. Hardware suggestions & resource estimation

Wearable node (ESP32 + MPU6050)

- Cost: low (~$6–$12)

- Memory: ESP32 (520 KB SRAM) — limited, not enough for typical LSTM; use ESP32 as streamer to gateway, not for heavy inference.

- Power: depends on sampling and WiFi usage.

Gateway (Raspberry Pi 4)

- Capabilities: can run TFlite, PyTorch (CPU), or ONNX

- Inference speed (estimate): LSTM (512×6, hidden=128) → ~10–100 ms on Pi 4 CPU depending on model and quantization.

Edge GPU (optional)

- Jetson Nano: faster inference, useful if processing multiple wearables.

9. API / MQTT topic structure (recommended)

- Telemetry streaming (if needed): device/<id>/telemetry
Payload: JSON or compact binary chunks (timestamp + raw sensor buffer)

- Window publish (preferred): device/<id>/window
Payload: Base64 or binary of 512×6 float32 values — smaller than continuous streaming.

- Event publish: device/<id>/event
Payload: {timestamp, p_fall, decision, window_index, meta}

- Commands to device (OTA, config): device/<id>/cmd
Payload: {cmd:"update", url:"..."}

10. Evaluation protocol (for paper)

- Subject-wise train/test split (no leakage): ensure no subject in both sets.

- Metrics: Accuracy, Precision, Recall, F1, Confusion Matrix — report per-window and also per-event (if aggregating windows into events).

- Latency & Memory: Measure inference latency, model memory footprint, CPU utilization on gateway hardware.

- Robustness tests: Add noise, sensor displacement, different sampling rates; report performance drop.

- Ablation study:

  - LSTM vs 1D-CNN vs hybrid CNN-LSTM

  - Window size variations (1s, 2.56s, 3s)

  - Overlap variants (25%, 50%, 75%)

  - Class balancing techniques (oversample, weighted loss)

- 11. Example architecture text you can paste in paper (ready-made paragraph)

- System Architecture. The proposed IoT fall detection system (Figure X) consists of wearable IMU sensor nodes, an inference gateway, a communication backbone, and a cloud-based monitoring/notification subsystem. Each wearable node (MPU6050 + ESP32) samples accelerometer and gyroscope data at 200 Hz and transmits fixed-length windows (2.56 s, 512 samples) to the gateway. The gateway performs per-window normalization and runs a lightweight LSTM classifier (input: 512×6; LSTM hidden size 128) exported to TensorFlow Lite for efficient inference. A fall decision is issued when the model’s fall probability exceeds a tuned threshold and confirmed via short-term voting to reduce false alarms. Confirmed events are broadcast via MQTT to a cloud backend where they are logged and used to trigger notifications (SMS/push). The modular design supports full edge deployment (inference on the gateway) or hybrid deployment (model served in the cloud) depending on available compute and power constraints.

12. Figure captions / table captions (ready copy)

Figure X. System architecture for the IoT-based fall detection pipeline. Wearable IMU nodes stream motion windows to an edge gateway for preprocessing and LSTM-based inference. Detected fall events generate local alerts and cloud notifications.

Table Y. Implementation and resource estimates for different deployment targets (ESP32 streamer + Raspberry Pi inference vs Raspberry Pi only vs cloud-only).

13. Practical next-steps to demonstrate a live IoT prototype (short checklist)

- Build wearable: ESP32 + MPU6050, flashing sampling code (send windows via MQTT).

- Setup gateway: Raspberry Pi subscribe to device/+/window, implement normalization and load TFLite model.

- Test loop: send recorded SisFall windows and verify gateway detects falls (compare to offline labels).

- Add UI & alerts: simple web dashboard + Twilio SMS for notifications.

- Measure latency, CPU, and battery life. Document in the paper.

14. Security, ethics, privacy (short paragraph for paper)

The proposed system minimizes privacy risks by transmitting only short, application-specific feature windows and event notifications rather than continuous raw audio/video. Communication is protected using TLS, devices are authenticated using per-device tokens, and cloud-stored data is encrypted at rest. For deployment on human subjects, we recommend institutional review and informed consent, and to limit personally identifiable information (PII) in telemetry and logs.

15. Closing — Suggested figure/file attachments for submission

- Figure 1: High-level system architecture (block diagram).

- Figure 2: LSTM model architecture (512×6 → LSTM 128 → Dropout → Dense).

- Figure 3: Confusion matrix heatmap (evaluation).

- Table 1: Performance comparison (LSTM vs baselines).

- Table 2: Device resource & latency estimates for deployment targets.